SUBJECT:	HONEST COMPANY PLAYA VISTA OFFICE

ADDRESS: 12121 BLUFF CREEK DRIVE

Drawings Dated:  08/11/2026

Drawing Reference: E0.01 TO E6.7-2

Addendum: None

BASE BID								$0.00

Inclusions:

- Power Distribution:

- Provide and install (3) 100A/3P and (1) 125A/3P circuit breaker on existing panel boards as shown on single line diagram.

- Provide (2) sub-meter as shown on single line diagram.

- Provide and install new panel boards 5L1A2LAB, 5L1A2MDF, 5L1B1A and 5L1B2A  as shown on single line diagram.

- Demo existing feeder that feeds existing panel LPAA and LPBB

- Provide and install (1) 1 1/4” C EMT with 4#2, 1#8 from existing panel 5L1A2 to new panel 5L1A2LAB (65LF).

- Provide and install (1) 1 1/4” C EMT with 4#2, 1#8 from existing panel 5L1B1 to new panel 5L1B1A (10LF).

- Provide and install (1) 1 1/4” C EMT with 4#2, 1#8 from existing panel 5L1B2 to new panel 5L1B2A (10LF).

- Provide and install (1) 1 1/2” C EMT with 4#1, 1#6 from existing panel 5L1A2 to new panel 5L1A2MDF (70LF).

- Lighting Plan:

- Provide and install lighting fixtures as shown on lighting plan.

- Provide and install lighting control devices as shown on lighting plan.

- Provide title 24 acceptance certification.

- Power Plan:

- Provide and install power devices with branch wiring as shown on plans.

- Provide and install poke thru devices as shown on plans.

- Provide and install disconnect switches for mechanical devices as shown on plans.

- LV, AV and SEC Rough:

- Provide and install back box and conduit stub with pull string for low voltage devices as shown on plans.

- Provide and install back box and conduit stub with pull string for security devices as shown on plans.

- Provide and install back box and conduit stub with pull string for security devices as shown on plans.

- Provide and install cable tray in the corridors as shown on plans. All cable trays inside the IDF and MDF will be provided and installed by LV contractor.

- Temp power and lighting:

- Provide temporary power and lighting.

- Clarifications:

- All work to be performed during regular working hours.

- Low voltage devices, wiring and termination are excluded.

- Security devices, wiring and termination are excluded.

- AV devices, wiring and termination are excluded.

- Fire alarm devices, conduit, wiring and termination are excluded.

- Parking fees are excluded. See alternate adder.

- Safe off and demolition is excluded.

Project Specific Exclusions:

- Electrical Permits

- Engineering and plan check fees

- Utility company charges or fees

- Temporary power and lighting

- Replace or upgrade existing panel boards.

- Low voltage devices, wiring and termination

- Security devices, conduit, wiring and termination

- Fire alarm devices, conduit, wiring and termination

- Access panels

- Overtime/ Shiftwork

- Unforeseen or bringing existing conditions into current code compliance

- Installing roof flashings, sealing of roof penetrations or any waterproofing

- Structural supports or structural engineering for any type of equipment

- Accelerated or compressed working schedule

- Concrete saw cutting, breaking removal and repair

- X-ray or Sonar of surfaces for penetration

- Patching and painting of all surfaces

- Poured and formed concrete or asphalt

- Demolition and removal or cutting walls

- Mechanical controls, panels, relays, VFDs, control conduit and wire

- Temporary task lighting, equipment connections or generators

- Removing trash, excavation spoils or demolished derbies off-site

- Special inspection fees including after normal work hour inspections

- Custom fixture colors not listed on this bid set of documents

- Exit or emergency lighting not shown on electrical drawings

- Vapor barrier seal-offs, infection and/or dust control

- Providing or installing ceiling seismic wires

- CAD As-builts, BIM or AutoCAD detailing

- Finished surface protection or covers

- Ceiling, wall or floor access panels